Set-: II

Q. Paper Code:

D.Y.PATIL COLLEGE OF ENGINEERING & TECHNOLOGY

KASABA BAWADA KOLHAPUR-416006

(An Autonomous Institute)

T. Y. B. Tech CIVIL

END SEMESTER EXAMINATION (ESE), Nov. – 2022

Course Name: Theory of structures      Course Code: 201CEL301

Seat No:

Day and Date: Tuesday, 6/12/2022

Time:  10 am to 12 pm 							 Max. Marks- 50

Instructions:

- All Questions 1is compulsory.

- Figure to the right indicate full marks.

- Assume suitable data if necessary

| BT | CO’s | Q.No. |  |  | Marks |
| --- | --- | --- | --- | --- | --- |
|  |  | Q.1 | All Questions are compulsory |  | 20 |
| 3 | CO1 | a | Find Static and kinematic indeterminacy of structures (i) (ii) |  | 4 |
| 3 | CO2 | b | Calculate support reaction at B for propped cantilever shown below use consistent deformation method, EI = 1x 107 N-m2 B 100 KN/m A 10mm Gap 2 m |  | 8 |
| 3 | CO2 | c | Calculate all support moment for continuous beam shown below draw BMD use Clapeyron’s TMT. Take EI = Constant 100 KN 25 KN/m C B A 3m 3m 4m |  | 8 |
|  |  | Q.2 | All Questions are compulsory |  | 10 |
| 3 | CO3 | a a | Calculate all support moments for continuous beam shown below use Slope deflection method 48 KN 45 KN C A B 2m 4m 3m 1m OR Calculate all support moments for continuous beam shown below use slope deflection method 90KN 60KN/m C B A 1m 5m 2m |  |  |
|  |  | Q.3 | All Questions are compulsory |  | 10 |
| 3 | CO3 | a a | Calculate all support moments for continuous beam shown below moment distribution method 60 KN 80 KN A B C 1m 3m 2m 1m 2I I OR Calculate all support moments for continuous beam shown below moment distribution method 72 KN C A B 30KN/m 2m 4m 6m 6m 3I 2I |  |  |
|  |  | Q.4 | Attempt any two out of three questions |  | 10 |
| 2 | CO4 | a | Define Maxwell’s reciprocal theorem with suitable example |  | 5 |
|  |  | b | Describe Castigliano’s theorem of minimum energy with suitable example |  | 5 |
|  |  | c | Describe unit load method |  | 5 |

**************